Computer System Organisation

DECEMBER 2024

Semester Questions Answers

- (a) Explain Von Neumann Architechture.

Von Neumann Architecture (also known as the Princeton Architecture) is a computer architecture model proposed by John von Neumann in 1945. It forms the foundation of most modern computers. The Von Neumann architecture is based on the idea that a single memory can hold both data and program instructions. The CPU fetches these instructions and executes them sequentially, unless a jump (branch) instruction changes the order.

### Main Components of Von Neumann Architecture

It consists of five basic parts:

- Input Unit

  - Used to enter data and instructions into the computer.

  - Example: Keyboard, Mouse.

- Output Unit

  - Displays or outputs processed data.

  - Example: Monitor, Printer.

- Memory Unit (Main Memory)

  - Stores data and instructions temporarily or permanently.

  - Divided into:

    - Primary Memory (RAM/ROM) – fast access storage.

    - Secondary Memory (Hard Disk, etc.) – long-term storage.

- Control Unit (CU)

  - Directs the operation of the processor.

  - Fetches instructions from memory, decodes them, and controls execution.

  - Acts like the “brain” of the CPU’s operation.

- Arithmetic and Logic Unit (ALU)

  - Performs all arithmetic operations (addition, subtraction, etc.) and logical operations (comparison, AND, OR, etc.).

2. (b) Discuss various types of Addressing Modes of Intel 8086.

### Addressing Modes of Intel 8086 Microprocessor

Definition:
Addressing modes specify how and from where the microprocessor fetches data or operands required for an instruction. In other words, they define the way an operand’s effective address (EA) is determined.

Intel 8086 supports different addressing modes to provide flexibility in programming, efficient memory access, and easy data manipulation.

## ⚙️ Classification of Addressing Modes in 8086

8086 has two broad categories of addressing modes:

- Data Addressing Modes

- Addressing modes for Branch or Control Transfer Instructions

## Data Addressing Modes

These are used to access data (operands) from registers or memory.

### (a) Immediate Addressing Mode

- The operand (data) is given directly in the instruction.

- No memory or register lookup is needed.

Syntax:
MOV AX, 1234H

Explanation:
Moves the immediate value 1234H into register AX.

Example in binary form:
Opcode + Data → all in one instruction.

### (b) Register Addressing Mode

- The operand is stored in a register, and the instruction specifies that register.

- No memory access is required.

Syntax:
MOV BX, AX

Explanation:
Copies the contents of register AX into register BX.

### (c) Direct Addressing Mode

- The effective address (EA) of the operand is given explicitly in the instruction.

- The data is stored in a specific memory location.

Syntax:
MOV AX, [1234H]

Explanation:
Moves the contents of memory location 1234H into AX.

Effective Address (EA): 1234H

### (d) Register Indirect Addressing Mode

- The address of the operand is held in a register pair.

- The operand is located in memory, but its address is specified indirectly through a register.

Syntax:
MOV AX, [BX]
MOV AX, [SI]
MOV AX, [DI]
MOV AX, [BP]

Explanation:
If BX = 2000H, then data is fetched from memory location 2000H and loaded into AX.

Effective Address (EA): Contents of the register (BX, BP, SI, DI)

### (e) Indexed Addressing Mode

- The effective address is obtained by adding a displacement (offset) to an index register.

Syntax:
MOV AX, [SI + 0500H]
or
MOV AX, [DI + 2000H]

Explanation:
EA = Contents of SI (or DI) + displacement (offset).

Used for array or table data.

### (f) Based Addressing Mode

- The effective address is formed by adding a base register (BX or BP) and a displacement.

Syntax:
MOV AX, [BX + 2000H]
or
MOV AX, [BP + 1000H]

Explanation:
EA = Base Register + Displacement

### (g) Based Indexed Addressing Mode

- The effective address is calculated by adding a base register, an index register, and optionally a displacement.

Syntax:
MOV AX, [BX + SI]
MOV AX, [BP + DI + 1000H]

Explanation:
EA = Base Register + Index Register (+ Displacement)

Used for accessing two-dimensional arrays or complex data structures.

### (h) Relative Based Indexed Addressing Mode

- Similar to based indexed, but a displacement is always added.

Syntax:
MOV AX, [BX + SI + 2000H]

Effective Address:
EA = Base Register + Index Register + Displacement

## Addressing Modes for Branch (Control Transfer) Instructions

Used for jump, call, loop, and interrupt instructions.

### (a) Intra-segment Direct Addressing Mode

- The target address is within the same code segment.

- The offset is specified directly in the instruction.

Syntax:
JMP SHORT LABEL

Explanation:
PC (Program Counter) = PC + displacement
The displacement is added to the current instruction pointer (IP).

### (b) Intra-segment Indirect Addressing Mode

- The target address is in the same segment, but its offset is stored in a register or memory.

Syntax:
JMP [BX]
CALL [DI]

Explanation:
The instruction pointer (IP) gets the value from the specified register or memory location.

### (c) Inter-segment Direct Addressing Mode

- The branch is made to a different segment.

- Both segment address and offset are given in the instruction.

Syntax:
CALL FAR PTR 5000H:2000H

Explanation:
CS (Code Segment) = 5000H
IP (Instruction Pointer) = 2000H

### (d) Inter-segment Indirect Addressing Mode

- Both segment and offset addresses are obtained indirectly from memory.

Syntax:
CALL FAR PTR [BX]

Explanation:
The memory location pointed by BX contains the offset and segment values.

- (a) Explain Restoring division algorithm for unsigned integer.

The Restoring Division Algorithm is a method for performing binary division of two unsigned integers, resulting in a quotient and a remainder. It operates iteratively through a series of shifts and subtractions.

Registers used:

- A (Accumulator/Remainder): Initially set to 0, this register holds the partial remainder during the process and the final remainder at the end. It's often one bit larger than the dividend to accommodate potential negative results from subtraction.

- Q (Quotient): Initially loaded with the dividend, this register stores the bits of the quotient as they are determined.

- M (Divisor): Holds the divisor throughout the algorithm.

## Registers Used

| Register | Meaning | Size |
| --- | --- | --- |
| A | Accumulator (Holds partial remainders) | Same size as divisor |
| Q | Quotient Register (Holds dividend and then quotient) | Same size as dividend |
| M | Divisor Register | Same size as divisor |
| n | Number of bits in Q (dividend) | — |

## Algorithm Steps:

Let:

- Dividend = Q

- Divisor = M

- A = 0 (initially)

We perform n iterations (where n = number of bits in Q).

### Step-by-Step Process

- Initialize:

  - A ← 0

  - Q ← Dividend

  - M ← Divisor

  - Count = number of bits in Q

- Repeat n times:

  - Shift Left:
Perform a left shift on (A, Q) together.

    - The most significant bit (MSB) of Q moves into the least significant bit (LSB) of A.

    - Equivalent to A:Q = A:Q << 1

  - Subtract Divisor:
A = A - M

  - Check Sign of A:

    - If A ≥ 0, then:

      - Set the least significant bit (LSB) of Q to 1.

    - Else (A < 0):

      - Restore the previous value of A: A = A + M

      - Set the LSB of Q to 0.

- End of Loop:

  - After n iterations:

    - Q contains the quotient.

    - A contains the remainder.

- (b) Multiply (-9) and (-13) using Booth’s algorithm.

Step 1: Binary Representation

First, represent the numbers -9 and -13 in 8-bit two's complement form.

- M = -9: The binary for 9 is 00001001. Inverting the bits gives 11110110, and adding 1 results in 11110111.

- Q = -13: The binary for 13 is 00001101. Inverting the bits gives 11110010, and adding 1 results in 11110011.

- -M: This will be the two's complement of M, which is the binary for +9: 00001001.

- The initial values for the algorithm are:

  - A = 00000000

  - Q = 11110011

  - Q_1 = 0

  - The number of iterations is n = 8 (the number of bits).

Step 2: Perform Booth’s Algorithm

The process involves checking the last two bits of the AQ register, 𝑄0𝑄1

.

| Iteration | A | Q | Q_1 | 𝑄0𝑄1 | Operation | A (new) | Q (new) | Q_1 (new) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | 00000000 | 11110011 | 0 | - | - | - | - | - |
| 1 | 00000000 | 11110011 | 0 | 10 | A = A - M | 00001001 | 11110011 | 0 |
|  |  |  |  |  | Arithmetic Right Shift | 00000100 | 11111001 | 1 |
| 2 | 00000100 | 11111001 | 1 | 11 | No Operation | 00000100 | 11111001 | 1 |
|  |  |  |  |  | Arithmetic Right Shift | 00000010 | 01111100 | 1 |
| 3 | 00000010 | 01111100 | 1 | 01 | A = A + M | 11111001 | 01111100 | 1 |
|  |  |  |  |  | Arithmetic Right Shift | 11111100 | 10111110 | 0 |
| 4 | 11111100 | 10111110 | 0 | 00 | No Operation | 11111100 | 10111110 | 0 |
|  |  |  |  |  | Arithmetic Right Shift | 11111110 | 01011111 | 0 |
| 5 | 11111110 | 01011111 | 0 | 10 | A = A - M | 00000111 | 01011111 | 0 |
|  |  |  |  |  | Arithmetic Right Shift | 00000011 | 10101111 | 1 |
| 6 | 00000011 | 10101111 | 1 | 11 | No Operation | 00000011 | 10101111 | 1 |
|  |  |  |  |  | Arithmetic Right Shift | 00000001 | 11010111 | 1 |
| 7 | 00000001 | 11010111 | 1 | 11 | No Operation | 00000001 | 11010111 | 1 |
|  |  |  |  |  | Arithmetic Right Shift | 00000000 | 11101011 | 1 |
| 8 | 00000000 | 11101011 | 1 | 11 | No Operation | 00000000 | 11101011 | 1 |
|  |  |  |  |  | Arithmetic Right Shift | 00000000 | 01110101 | 1 |

Answer:

The final product is the 16-bit number in the AQ registers after 8 shifts. The result is 0000000001110101. This corresponds to the decimal number 117 (20+22+24+25+26 =1+4+16+32+64=117).

Since multiplying a negative number by a negative number results in a positive number, this is the correct answer for (-9)×(-13).

## (a) Block Diagram of 8086 Architecture

The Intel 8086 is a 16-bit microprocessor introduced by Intel in 1978.
It has a 20-bit address bus and a 16-bit data bus, which allows it to directly access 1 MB (2²⁰ = 1,048,576 bytes) of memory.

To increase efficiency, Intel divided the internal architecture into two main functional units:

- Bus Interface Unit (BIU)

- Execution Unit (EU)

## Bus Interface Unit (BIU)

### 🔸 Purpose:

The BIU handles all data and address transfers between memory, input/output devices, and the microprocessor.
    It fetches instructions from memory and queues them for the Execution Unit.

### 🔹 Functions of BIU:

- Generates the 20-bit physical address for memory access.

- Fetches instructions from memory in advance.

- Stores the fetched instructions in a queue (pipelining).

- Handles data read/write from/to memory or I/O ports.

### 🔹 Components of BIU:

#### (a) Segment Registers (4 × 16-bit):

These are used to form a 20-bit physical address (Segment × 16 + Offset).

| Register | Function |
| --- | --- |
| CS (Code Segment) | Holds base address of the current program (instruction code). |
| DS (Data Segment) | Holds base address of data variables. |
| SS (Stack Segment) | Holds base address of the stack (used for push/pop). |
| ES (Extra Segment) | Used for additional data storage (e.g., string operations). |

Example of Physical Address Calculation:

Physical Address = (Segment Register × 10H) + Offset

e.g.  CS = 2000H, IP = 3000H

Physical Address = 2000H × 10H + 3000H = 23000H

#### (b) Instruction Pointer (IP):

- Holds the offset address of the next instruction to be executed in the code segment.

#### (c) Instruction Queue (6 bytes):

- The BIU prefetches up to 6 bytes of instructions and stores them in the queue.

- While the EU executes one instruction, the BIU fetches the next — this is called pipelining, improving speed.

#### (d) Address Generation Unit:

- Calculates physical addresses using segment + offset.

- Sends these addresses to the memory through the address bus.

### 🧠 BIU Key Advantage:

Because of the instruction queue, the 8086 can fetch and execute instructions simultaneously, which significantly increases processing speed.

## Execution Unit (EU)

### 🔸 Purpose:

The EU is responsible for decoding and executing instructions fetched by the BIU.

### 🔹 Functions of EU:

- Takes instructions from the queue (BIU).

- Decodes the instructions.

- Performs arithmetic and logic operations.

- Controls program flow (branching, loops, interrupts).

- Sends requests to BIU for memory access when needed.

### 🔹 Components of EU:

#### (a) Arithmetic and Logic Unit (ALU):

- Performs all arithmetic operations (add, subtract, increment, decrement)
and logical operations (AND, OR, XOR, NOT, compare).

#### (b) General Purpose Registers (16-bit):

Used to store data temporarily during execution.

| Register | Also usable as 8-bit | Function |
| --- | --- | --- |
| AX | AH / AL | Accumulator register (used in arithmetic, logic, and I/O). |
| BX | BH / BL | Base register (used for addressing). |
| CX | CH / CL | Count register (used in loops, shifts). |
| DX | DH / DL | Data register (used in multiplication/division, I/O). |

#### (c) Pointer and Index Registers (16-bit):

Used for memory addressing and stack operations.

| Register | Function |
| --- | --- |
| SP (Stack Pointer) | Points to top of the stack in the stack segment. |
| BP (Base Pointer) | Used to access data in stack segment. |
| SI (Source Index) | Used in string operations (source). |
| DI (Destination Index) | Used in string operations (destination). |

#### (d) Flag Register (16-bit):

Stores status flags that reflect results of operations and control processor behavior.

| Type | Flags | Meaning |
| --- | --- | --- |
| Status Flags | CF, ZF, SF, OF, PF, AF | Indicate results (carry, zero, sign, overflow, parity, auxiliary). |
| Control Flags | DF, IF, TF | Control processor operations (direction, interrupt enable, trap). |

#### (e) Instruction Decoder and Control Circuitry:

- Decodes instructions from the queue.

- Generates control signals to drive the ALU, registers, and buses.

## 4. (b) Explain different types of interrupts.

## What is an Interrupt?

An interrupt is a signal that causes the 8086 microprocessor to temporarily stop the execution of its current program and start executing a special program called the Interrupt Service Routine (ISR) or Interrupt Handler. After servicing the interrupt, the CPU returns to the main program and continues execution from where it left off.

## Types of Interrupts in 8086

Interrupts in the 8086 microprocessor are broadly classified as:

- Hardware Interrupts

- Software Interrupts

### 🧩 1. Hardware Interrupts

These are generated by external hardware devices to request CPU attention.

There are two hardware interrupt inputs in 8086:

- INTR (Interrupt Request)

- NMI (Non-Maskable Interrupt)

#### (a) INTR – Maskable Interrupt

- It is a general-purpose interrupt line.

- The CPU checks INTR at the end of each instruction cycle.

- It can be enabled or disabled using the Interrupt Flag (IF).

  - STI instruction → sets IF (enables interrupt)

  - CLI instruction → clears IF (disables interrupt)

- When INTR is activated:

  - The CPU completes the current instruction.

  - Sends an Interrupt Acknowledge (INTA) signal to the device.

  - The device places the interrupt type number on the data bus.

  - CPU uses that number to locate the ISR address from the Interrupt Vector Table (IVT).

#### (b) NMI – Non-Maskable Interrupt

- NMI is a high-priority interrupt.

- It cannot be disabled by any software instruction.

- It is used for emergency or critical events, such as:

  - Power failure

  - Memory parity error

  - Hardware fault

- NMI uses a fixed interrupt vector of type 2 (INT 2).

### 💻 2. Software Interrupts

These are initiated by the programmer using the INT instruction.

#### Syntax:  INT n

#### where n is the interrupt type number (0–255).

#### How it works:

- When INT n is executed:

  - The CPU pushes FLAGS, CS, and IP onto the stack.

  - Clears the IF and TF flags (to disable further interrupts).

  - Fetches the ISR address from the Interrupt Vector Table (IVT).

  - Jumps to that ISR to handle the interrupt.

  - After ISR, returns to the main program using IRET.

#### Examples:

| Instruction | Type No. | Description |
| --- | --- | --- |
| INT 0 | 0 | Divide-by-zero error |
| INT 1 | 1 | Single-step (used for debugging) |
| INT 3 | 3 | Breakpoint interrupt |
| INT 4 | 4 | Overflow interrupt |
| INT 21H | 33 | DOS function interrupt (used in assembly programming for I/O operations) |

5.Calculate the number of page faults for the following reference string with frame size as 3 by using LRU and FIFO page replacement algorithm: 5,0,2,1,0,3,0,2,4,3,0,3,2,1,3,0,1,5

# FIFO (3 frames)

| Step | Reference | Frame1 | Frame2 | Frame3 | Result |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5 | - | - | Fault |
| 2 | 0 | 5 | 0 | - | Fault |
| 3 | 2 | 5 | 0 | 2 | Fault |
| 4 | 1 | 1 | 0 | 2 | Fault |
| 5 | 0 | 1 | 0 | 2 | Hit |
| 6 | 3 | 1 | 3 | 2 | Fault |
| 7 | 0 | 1 | 3 | 0 | Fault |
| 8 | 2 | 2 | 3 | 0 | Fault |
| 9 | 4 | 2 | 4 | 0 | Fault |
| 10 | 3 | 2 | 4 | 3 | Fault |
| 11 | 0 | 0 | 4 | 3 | Fault |
| 12 | 3 | 0 | 4 | 3 | Hit |
| 13 | 2 | 0 | 2 | 3 | Fault |
| 14 | 1 | 0 | 2 | 1 | Fault |
| 15 | 3 | 3 | 2 | 1 | Fault |
| 16 | 0 | 3 | 0 | 1 | Fault |
| 17 | 1 | 3 | 0 | 1 | Hit |
| 18 | 5 | 3 | 0 | 5 | Fault |

Total references: 18
FIFO page faults: 15 (hits = 3)

# LRU (3 frames)

| Step | Reference | Frame1 (LRU) | Frame2 | Frame3 (MRU) | Result |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5 | - | - | Fault |
| 2 | 0 | 5 | 0 | - | Fault |
| 3 | 2 | 5 | 0 | 2 | Fault |
| 4 | 1 | 5 | 0 | 1 | Fault |
| 5 | 0 | 5 | 1 | 0 | Hit |
| 6 | 3 | 1 | 0 | 3 | Fault |
| 7 | 0 | 1 | 3 | 0 | Hit |
| 8 | 2 | 3 | 0 | 2 | Fault |
| 9 | 4 | 0 | 2 | 4 | Fault |
| 10 | 3 | 2 | 4 | 3 | Fault |
| 11 | 0 | 4 | 3 | 0 | Fault |
| 12 | 3 | 4 | 0 | 3 | Hit |
| 13 | 2 | 0 | 3 | 2 | Fault |
| 14 | 1 | 3 | 2 | 1 | Fault |
| 15 | 3 | 2 | 1 | 3 | Hit |
| 16 | 0 | 1 | 3 | 0 | Fault |
| 17 | 1 | 3 | 0 | 1 | Hit |
| 18 | 5 | 0 | 1 | 5 | Fault |

(Frame columns show order LRU → MRU for LRU algorithm; “-” means unused)

Total references: 18
LRU page faults: 13 (hits = 5)

  - (a) Compare between direct mapped, set associative and associative cache.

| Feature | Direct Mapped Cache | Set Associative Cache | Fully Associative Cache |
| --- | --- | --- | --- |
| Definition | Each memory block maps to exactly one cache line. | Each memory block maps to a set; it can be placed in any line within that set. | Any memory block can be placed in any cache line. |
| Mapping Function | Cache line = (Block address) mod (No. of cache lines) | Cache set = (Block address) mod (No. of sets) | No fixed mapping (anywhere in cache). |
| Structure | 1 line per set (1-way associative). | k lines per set (k-way associative). | 1 set with all lines (n-way associative). |
| Tag Comparison | Only one tag compared — very simple. | Multiple tags (within a set) compared simultaneously. | All tags in cache compared simultaneously. |
| Access Speed | Fastest (only one possible location). | Moderate (check a few locations). | Slowest (check all locations). |
| Hardware Complexity | Simple hardware design. | Moderate complexity. | Very high complexity (needs associative search hardware). |
| Miss Rate | Highest (conflict misses are common). | Moderate (some flexibility reduces misses). | Lowest (no conflict misses). |
| Replacement Policy | Not needed (only one choice). | Required within the set (LRU, FIFO, Random). | Required (LRU, FIFO, Random). |
| Flexibility in Block Placement | Least flexible (only one location possible). | Medium flexibility (limited to a set). | Most flexible (can go anywhere). |
| Cost | Lowest (simple and cheap). | Moderate (balance of cost and performance). | Highest (complex and expensive). |
| Example | Block 0, 4, 8, 12 → Line 0; Block 1, 5, 9, 13 → Line 1, etc. | 2-way: Block 0 & 4 can go in Set 0; Block 1 & 5 in Set 1. | Any block (0–15) can go into any cache line. |
| Applications | Simple processors, embedded systems. | General-purpose CPUs (most common). | High-performance systems (rare in practice). |

  - (b) Differentiate Between Write Through and Write Back Methods.

| S.No. | Write Through Method | Write Back Method |
| --- | --- | --- |
| 1 | In this method main memory is updated with every memory write operation as well as cache memory is updated in parallel if it contains the word at the specified address. | In this method only cache location is updated during write operation. |
| 2 | Main memory always contains same data as cache. | Main memory and cache memory may have different data. |
| 3 | Number of memory write operation in a typical program is more. | Number of memory write operation in a typical program is less |
| 4 | When I/O device communicated through DMA would receive most recent data. | When I/O device communicated through DMA would not receive most recent data. |
| 5 | It is a process of writing cache and main memory simultaneously. | It is a process of writing cache and data is removed from cache, first copied to main memory. |

  - (a) Differentiate between Fixed point and Floating point representation.

| Basis of Comparison | Fixed Point Representation | Floating Point Representation |
| --- | --- | --- |
| Definition | Numbers are represented with a fixed number of digits before and after the decimal (or binary) point. | Numbers are represented with a variable number of digits before and after the decimal point, using mantissa and exponent. |
| Format | The position of the decimal (binary) point is fixed. | The position of the decimal (binary) point is variable and indicated by the exponent. |
| Representation form | Integer part + fractional part. | Sign bit + exponent + mantissa (e.g., IEEE 754 format). |
| Range | Has a limited range of representable numbers. | Has a very wide range due to the use of exponent. |
| Precision | Provides higher precision but smaller range. | Provides higher range but precision depends on mantissa size. |
| Complexity | Simple to implement in hardware. | More complex to implement (requires exponent handling and normalization). |
| Speed | Faster arithmetic operations. | Slower arithmetic operations due to normalization and rounding. |
| Example (decimal) | 123.45 → represented as 12345 with two fixed decimal places. | ( 1.2345 \times 10^2 ) → represented using mantissa 1.2345 and exponent 2. |
| Use Cases | Suitable for applications where range is small and precision is more important (e.g., microcontrollers, embedded systems). | Suitable for scientific and engineering calculations where large ranges are needed. |

(b) Let's represent –75 as an 8-bit 2’s complement number step by step?

### Step 1: Write +75 in binary (unsigned 8-bit)

We first convert 75 to binary.

| Calculation | Binary |
| --- | --- |
| 75 ÷ 2 = 37 remainder 1 | (LSB) |
| 37 ÷ 2 = 18 remainder 1 |  |
| 18 ÷ 2 = 9 remainder 0 |  |
| 9 ÷ 2 = 4 remainder 1 |  |
| 4 ÷ 2 = 2 remainder 0 |  |
| 2 ÷ 2 = 1 remainder 0 |  |
| 1 ÷ 2 = 0 remainder 1 | (MSB) |

So, reading from MSB → LSB:

75₁₀ = 01001011₂

### Step 2: Find the 1’s complement

Invert all bits:

01001011 → 10110100

### Step 3: Add 1 to get the 2’s complement

10110100

+         1

-----------

10110101

✅ Final Answer:

[
\boxed{-75_{10} = 10110101_2}
]

  - (B) What is Direct Memory Access?

DMA (Direct Memory Access) — Explained Simply

Definition:
Direct Memory Access (DMA) is a data transfer technique in which data is transferred directly between memory and an input/output (I/O) device without involving the CPU for each byte or word of data.

### 🔹 Key Idea

In normal data transfer, the CPU reads data from an I/O device and then writes it to memory — this consumes CPU time.
With DMA, a special hardware component called the DMA Controller (DMAC) handles the transfer, freeing the CPU to perform other tasks.

### 🔹 How DMA Works

- The CPU initializes the DMA controller by giving it:

  - The starting address of data in memory

  - The device address

  - The number of bytes to transfer

  - The direction (read/write)

- The DMA controller takes over the system bus (temporarily) and directly transfers data between memory and the I/O device.

- After the transfer is complete, the DMA controller sends an interrupt signal to the CPU indicating completion.

### 🔹 Advantages

- Faster data transfer compared to CPU-controlled I/O

- CPU is free for other processing during data transfer

- Efficient for large block transfers (e.g., disk, sound card, network operations)

### 🔹 Disadvantages

- More hardware complexity and cost (requires DMA controller)

- Possible bus conflicts between DMA and CPU

- Not efficient for small data transfers

### 🔹 Example

When copying a large file from a hard disk to RAM, the DMA controller moves the data directly — instead of the CPU handling each byte.

  - (A) What is Pipelining?

Pipelining is a technique used in CPU design to increase instruction throughput — that is, the number of instructions executed per unit time.

It allows overlapping execution of multiple instructions by dividing the instruction execution process into separate stages, where each stage performs a part of the instruction.

### 🔹 Example:

The stages of a typical instruction pipeline are:

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write Back

While one instruction is in the Execute stage, another can be in Decode, and a third can be in Fetch, all simultaneously.

## ⚙️ Advantages of Pipelining

- Increases CPU throughput (more instructions executed per unit time)

- Improves performance without increasing clock speed

- Efficient use of CPU resources

  - (b) Hazards in Pipelining

While pipelining improves performance, it can face hazards — situations that delay or disrupt instruction flow.

### 🔸 1. Structural Hazards

- Occur when two or more instructions require the same hardware resource at the same time.

- Example: If both the Fetch and Memory stages need access to memory simultaneously, a conflict occurs.

Solution: Use separate instruction and data memory (Harvard architecture) or add more hardware resources.

### 🔸 2. Data Hazards

- Occur when instructions depend on the results of previous instructions that have not yet completed.

#### Types:

| Type | Example | Description |
| --- | --- | --- |
| RAW (Read After Write) | I1: R1 ← R2 + R3 I2: R4 ← R1 + R5 | I2 needs R1 before I1 writes it → most common hazard |
| WAR (Write After Read) | I1: R4 ← R1 + R2 I2: R1 ← R3 + R5 | I2 writes R1 before I1 has read it |
| WAW (Write After Write) | I1: R1 ← R2 + R3 I2: R1 ← R4 + R5 | Both write to same register R1 in overlapping stages |

Solutions:

- Data forwarding (bypassing)

- Instruction scheduling or pipeline stalls (inserting NOPs)

### 🔸 3. Control Hazards (Branch Hazards)

- Occur due to branch or jump instructions that change the program flow.

- The next instruction to fetch is uncertain until the branch is resolved.

Example:

BEQ R1, R2, LABEL

Until we know if the branch is taken, we cannot know the next instruction.

Solutions:

- Branch prediction

- Branch delay slots

- Speculative execution

  - (a) Write short notes on Vector Processor

A vector processor is a type of central processing unit (CPU) that can perform a single instruction on multiple data elements simultaneously, a process called vector processing. This contrasts with scalar processors, which operate on one data item at a time. Vector processors are ideal for data-intensive tasks like scientific computing, image processing, and artificial intelligence, where the same operation is repeated on large arrays of data. Key components include special vector registers to hold data and vector instructions that operate on these entire vectors.

Key features

- Parallel processing: They process multiple data elements in parallel, which greatly increases performance for specific workloads.

- Vector instructions: Instructions are designed to operate on a whole vector (a one-dimensional array of data) rather than individual elements, which can reduce the number of instructions needed.

- Vector registers: These are special registers that can store entire vectors of data, enabling simultaneous operations on all elements at once.

- Pipelining: Vector processors often use pipelining to overlap the execution of multiple instructions, further boosting performance.

Applications

- Scientific and numerical computing: Used for complex calculations in fields like simulations and weather forecasting.

- Image and video processing: Handles the large datasets involved in image manipulation and rendering.

- Artificial intelligence and machine learning: Accelerates the parallel computations required for AI algorithms.

- Other applications: Also used in fields like medical diagnosis and seismic data analysis.

(b) Short Notes on RISC Pipeline.

RISC pipelining is a technique used in Reduced Instruction Set Computer (RISC) processors to improve performance by breaking down instruction execution into distinct stages, like an assembly line. This allows the processor to work on different parts of multiple instructions simultaneously, increasing throughput. The classic five stages are fetch, decode, execute, memory access, and writeback, with each stage handling a different instruction at any given time.

How it works

- Instruction stages: Each instruction is broken down into a series of simple, sequential steps or stages.

- Concurrent processing: While one instruction is in the "execute" stage, the next one can be in the "decode" stage, and another in the "fetch" stage.

- Throughput: This concurrency, where the processor can complete one instruction per clock cycle on average, significantly increases the overall performance of the system.

- Analogy: The process is often compared to doing laundry. While the first load is in the dryer, the second is in the washer, and the third is being prepared to be put in the washer.

Key characteristics and challenges

- Simplified hardware: The use of a reduced and simple instruction set makes the hardware simpler to design and more efficient.

- Data dependencies: A common issue is when one instruction needs the result of a previous one, which can cause a stall or "bubble" in the pipeline until the data is ready.

- Branch instructions: Conditional branching can disrupt the flow of instructions, as the processor may not know which instruction to fetch next until the branch condition is resolved.

- Solutions: RISC architectures use techniques like compiler reordering, delayed branching, or branch prediction to minimize stalls caused by dependencies and branches.

(c) Short note on Cache hit and Cache miss.

A cache is a high-speed data storage layer that temporarily stores frequently accessed data to serve future requests faster. When a system requests data, it first checks the cache. The result of this check determines whether a cache hit or a cache miss occurs.

Cache hit

A cache hit happens when the requested data is found in the cache. Since the cache is much faster than the main memory or primary data source, the data can be retrieved quickly, which significantly improves overall system performance.

How it works

- The CPU, web browser, or application first looks for the data in the cache.

- If a copy of the data is present, it is served directly from the cache.

- This fast retrieval minimizes latency and reduces the load on the slower main storage.

Cache miss

A cache miss occurs when the data requested is not in the cache. When this happens, the system must retrieve the data from a slower, lower-level storage location, such as main memory (RAM) or a hard disk drive.

How it works

- The system searches the cache for the requested data and does not find it.

- The system then accesses the slower main memory to find and fetch the data.

- Once retrieved, the data is also copied into the cache, so future requests for the same data will result in a cache hit.

Common types of cache misses

- Compulsory (Cold) miss: Occurs on the first-ever access to a data block, as the cache is initially empty.

- Capacity miss: Happens when the cache is not large enough to hold all the data required by a program. Older data must be evicted to make space for new data.

- Conflict miss: Occurs in specific cache architectures when two or more data blocks compete for the same cache location, causing one to be replaced.

Importance and impact on performance

The ratio of cache hits to total requests is known as the cache hit ratio, which is a key measure of a cache's efficiency.

- High hit ratio: Indicates that the cache is effective, leading to faster data access and better performance. This is the desired state for most applications.

- High miss ratio: Implies the cache is not being used effectively, resulting in frequent, slower access to main memory. This increases latency and can create a performance bottleneck.